"To run the scripts, you need to have Python installed and install the following geospatial libraries via your terminal: pip install rasterio geopandas shapely numpy scipy"

1. Project Overview & Objective

This project aims to perform a high-precision, directional Annual Energy Production (AEP) analysis for a 1x1 km area in Adliswil, Switzerland. The methodology integrates macro-scale meteorological data with micro-scale topographical data to calculate the localized energy yield of a 300 kW wind turbine.

The core challenge addressed is the downscaling of regional wind data to a highly granular level (0.5 m resolution) to account for the aerodynamic shading (wake effects) caused by local obstacles such as buildings and trees.

2. Data Sources & Parameters

- Topographical Data: normalized Digital Surface Model (nDSM) at 0.5 m resolution (nDSM_obstacles_2680-1237.tif).

Workflow Instruction: Run 01_create_ndsm.py first. This script subtracts the bare-earth terrain model (swissalti3d_2026_2680-1237_0.5_2056_5728.tif) from the surface model (swisssurface3d-raster_2024_2680-1237_0.5_2056_5728.tif) to generate the clean nDSM obstacle map (nDSM_obstacles_2680-1237.tif).

- Meteorological Data: Swiss Federal Office of Energy (SFOE) Wind Atlas at 50 m height (Windatlas_50.gpkg – download from https://opendata.swiss/en/dataset/windatlas-schweiz-jahresmittel-der-modellierten-windgeschwindigkeit-und-windrichtung-in-50-m-ho). This GeoPackage contains a 100 m vector grid with Weibull parameters (A, k) and frequency (%) for 12 directional wind sectors (0° to 330°).

Workflow Instruction: Run 02_extract_wind_data.py next. This script geographically clips the massive national Wind Atlas strictly to our study area, generating Adliswil_wind_sectors.gpkg for faster processing.

- Turbine Specifications:

  - Rated Power: 300 kW

  - Hub Height (Hhub): 29.0 m

  - Power Curve data interpolated from manufacturer specifications (Cut-in: 3 m/s, Rated: 11 m/s, Cut-out: 16 m/s).

3. Physical Principles & Mathematical Models

The computational pipeline relies on several aerodynamic and statistical principles to model the interaction between the wind and the local terrain.

Workflow Instruction: All the mathematical models and physics described below are executed by running the final simulation script: 03_calculate_directional_aep.py. Inputs required: nDSM_obstacles_2680-1237.tif (generated in Step 1) and Adliswil_wind_sectors.gpkg (generated in Step 2). Final Output: AEP_Adliswil_300kW_29m_WakeEffect.tif (The finalized energy production map ready for GIS software). Then drag and drop the AEP_Adliswil_300kW_29m_WakeEffect.tif into ArcGIS program to visualize the AEP across the region of Adliswil.

Of course all the physics and equations below may not be the most precise ones – they were used just to test:

3.1. Wind Shear Power Law

Wind speed increases with height due to reduced friction with the Earth's surface. The vertical wind profile is modeled using the empirical power law:

Vz=Vrefzzrefα

Where:

- Vz is the wind speed at height z.

- Vref is the reference wind speed at height zref (50 m).

- α is the wind shear exponent (set to 0.143 for standard surface roughness).

3.2. Effective Hub Height (Heff)

To account for the local blockage effect of obstacles, the physical hub height is reduced by the height of the obstacle (hobs) at that specific location. A safety threshold of 5.0 m is maintained:

Heff=maxHhub-hobs,5.0

3.3. Directional Wake Effect (Downwind Propagation)

An obstacle does not only affect the wind directly above it but casts an aerodynamic shadow downwind. For a wind sector with angle θ, the shadow propagates in the θ+180∘ direction. The influence of the obstacle decays exponentially as the downwind distance (d) increases:

hshadow=hobs⋅e-λd

Where:

- λ is the decay factor (set to 0.015).

- The effective height for any pixel in the wake is determined by the maximum shadow cast by all upwind obstacles:

Heff=maxHhub-maxhlocal,hshadow,5.0

3.4. Localized Weibull Distribution

Wind speeds follow a Weibull probability distribution. Rather than adjusting the wind speed directly, the scale parameter (A) is dynamically adjusted for every 0.5 m pixel based on the local effective height relative to the reference height of 50 m:

Alocal=AbaseHeff50α

The shape parameter (k) remains constant. The Probability Density Function (PDF) for a given wind speed v is:

PDFv=kAlocalvAlocalk-1e-v/Alocalk